# Project report for Home Automation System

Maven Silicon Confidential

All the presentations, books, documents [hard copies and soft copies] labs and projects [source code] that you are using and developing as part of the training course are the proprietary work of Maven Silicon and it is fully protected under copyright and trade secret laws. You may not view, use, disclose, copy, or distribute the materials or any information except pursuant to a valid written license from Maven Silicon

# Table of Contents

- Student information                                                                                                            4

- Introduction                                                                                                                           4

- Project Specifications                                                                                                           5

3.1     Function Requirements                                                                                             5

3.2     Non-functions Requirements                                                                                   6

- Hardware Architecture                                                                                                         7

4.1 List of hardware components used                                                                              9

- Software Architecture                                                                                                          10

- Code                                                                                                                                        15

- Testing and results                                                                                                               18

- Future work                                                                                                                            	20

# 1. Student information

Student Name: Pradnyesh Diwan

University Name and Location: VIT University, Vellore, Tamil Nadu

Submission Date: 04-07-2025

# 2. Introduction

In modern homes, offices, and classrooms, automation has become an essential part of improving energy efficiency and user convenience. This project, titled "Smart Fan and Light Control System using ESP32", is designed to intelligently manage the operation of a fan and a light based on real-time environmental conditions and human presence.

This system is particularly relevant in scenarios where unnecessary energy consumption needs to be reduced without compromising comfort. For instance:

- Home automation: Automatically turning ON/OFF lights and fans when someone enters or leaves a room eliminates the need for manual switches and prevents energy wastage.

- Educational institutions: In classrooms, fans and lights often remain ON even when rooms are unoccupied. This system detects presence and ambient light to operate only when needed.

- Office spaces and public buildings: Automated control ensures optimal usage of electrical appliances during working hours and shuts them off when rooms are vacant or naturally lit.

- Hospitals and elderly care: Offers contactless operation, reducing the need for physical switches—useful for hygiene and accessibility.

The importance of this system lies in its contribution to energy conservation, automation, and user convenience:

- Energy Efficiency: By using sensors (ultrasonic, LDR, and DHT11), the system ensures that electrical devices operate only under necessary conditions, thereby reducing electricity bills and resource wastage.

- Smart Context Awareness: Unlike traditional systems, it considers multiple parameters (presence, temperature, and light) for decision-making, which mimics human reasoning

- Touchless Automation: Especially post-pandemic, reducing contact with surfaces has become a priority in many settings.

- Scalability: This modular system can be expanded further with IoT, scheduling, or remote control, making it future-ready for smart infrastructure.

# 3. Project Specifications

## 3.1 Function Requirements

The system must fulfil the following functional requirements to ensure intelligent and efficient control of electrical appliances based on environmental conditions and human presence.

3.1.1 Sensor-Based Monitoring

- The system shall detect the presence of a person using an ultrasonic sensor based on distance measurement.

- The system shall measure ambient light levels using an LDR (Light Dependent Resistor) to determine if the room is dark or sufficiently lit.

- The system shall monitor temperature and humidity using a DHT11 sensor to assess the thermal environment.

3.1.2 Automatic Fan Control

- The fan shall turn ON when a person is detected and the temperature exceeds 25°C.

- The fan shall turn OFF when no person is detected, regardless of the temperature value.

3.1.3 Automatic Light Control

- The light shall turn ON when a person is present and the ambient light is below a defined threshold (i.e., the room is dark).

- The light shall turn OFF when:

  - The room is adequately lit

  - No person is present.

3.1.4 System Decision Logic

- The ESP32 microcontroller shall continuously process sensor data in real-time.

- The relay module shall activate or deactivate the fan and light based on the logical conditions derived from sensor readings.

3.1.5 Energy Efficiency and Safety

- The system shall ensure minimal power usage by operating appliances only when required.

- Relay circuits must switch appliances within safe voltage and current ratings to prevent electrical hazards.

3.1.6 Provision for Future Expansion

- The system shall be modular and allow future integration of features such as:

  - IoT-based remote control and monitoring

  - OLED display for real-time data

  - Manual override switches or mobile app interface

# 3.2 Non-Function Requirements

Non-functional requirements define the quality attributes of the system and ensure it performs effectively, reliably, and securely under various conditions. These requirements focus on system performance, usability, maintainability, and scalability.

3.2.1 Performance

  - The system shall process sensor inputs and activate relays within 2 seconds to   ensure real-time response

  - The system shall support continuous operation without noticeable lag or delay in switching appliances.

3.2.2 Reliability

  - The system shall function consistently and accurately under stable power supply and typical indoor environmental conditions.

  - The system shall continue to operate for extended durations (e.g., 24 hours a day) without requiring manual resets.

3.2.3 Usability

  - The system shall be simple to operate, requiring no manual input from the user for regular operation.

  - The behavior of the system shall be intuitive and based entirely on environmental context (presence, light, temperature).

3.2.4 Maintainability

  - The system shall be designed using modular code and components so that updates or component replacements can be performed easily.

  - Sensor thresholds and logic parameters shall be easily adjustable in the source code for tuning performance.

3.2.5 Scalability

  - The system shall be expandable to support additional sensors (e.g., motion, gas, sound) or outputs (e.g., additional fans or lights).

  - The ESP32's onboard Wi-Fi and processing capability shall allow future IoT integration without hardware change.

3.2.6 Portability

  - The system shall be portable and deployable in various room types (home, classroom, office) with minor adjustments.

  - The design shall support power supply via standard 5V USB or 3.3V regulator, making it compatible with multiple deployment setups.

3.2.7 Safety

  - All electrical connections to the relay module shall be securely insulated to prevent short circuits or electric shock.

  - The system shall not operate beyond the rated limits of connected appliances to ensure safe operation.

# 4. Hardware Architecture

4.0.1 Board and Peripheral Interface Selection

Why ESP32 :

- Built-in Wi-Fi and Bluetooth for future IoT expansion

- Multiple GPIOs and analog input pins

- Low power consumption

- Cost-effective and reliable for prototyping and production

- Supports multiple interrupts and PWM outputs

Why 2-Channel Relay Module:

- Allows control of two AC devices (fan and light)

- Compatible with ESP32's 3.3V digital output

- Electrical isolation ensures safety

Why DHT11:

- Simple digital interface

- Sufficient accuracy for indoor temperature and humidity monitoring

Why Ultrasonic Sensor:

- Reliable for detecting human presence within 1–2 meters

- Works in dark/light conditions unlike IR sensors

Why LDR Sensor:

- Analog sensor for detecting ambient light level

- Simple voltage divider interface with ESP32

4.0.2 Ensuring Requirements Are Met

To ensure that the system meets the functional and non-functional requirements, the following steps were followed:

- Real-time Processing: ESP32 processes data every 2 seconds using efficient loops.

- Correct Logic Implementation: Fan control depends on both presence and temperature; light control depends on presence and darkness.

- Modular Code: The program is written in clearly separated functions for maintainability.

- Energy Efficiency: Devices operate only when needed based on sensor logic.

- Reliability: Components selected are known for stable performance in embedded systems.

- Safety Measures: All relay and sensor connections use proper resistors and opto-isolation where needed.

- Scalability: The ESP32 supports additional peripherals (e.g., OLED, IoT modules) for future improvements.

- Manual Testing: The system was tested under various combinations of light, presence, and temperature to validate response time and correctness.

4.0.3 Schematic

## 4.1 List of Hardware Components used

| Serial Number | Description of Hardware | Quantity | Remarks if any |
| --- | --- | --- | --- |
| 1. | ESP 32 | 1 | Microcontroller |
| 2. | DHT 11 Sensor | 1 | To check temperature and humidity |
| 3. | LDR Sensor | 1 | To check any other artificial light other than required for the project is present or not |
| 4. | 2 Channel Relay Module | 1 | To control light and a fan based on the input from the sensors |
| 5. | Ultrasonic Sensor | 1 | To check whether a person is present in the room |

# 5. Software Architecture

To design the smart fan and light control system efficiently, the project was divided into main functional blocks and sub-functions. Each block addresses a specific responsibility (e.g., sensing, decision-making, actuation), which translates into code functions during implementation.

| Function Name | Prototype | Purpose |
| --- | --- | --- |
| readTemperature | float readTemperature(); | Reads temperature data from the DHT11 sensor in °C. |
| readLDR | int readLDR(); | Reads ambient light intensity using the LDR (returns analog value). |
| getDistance | float getDistance(); | Measures the distance from an object using the ultrasonic sensor (in cm). |
| isPersonPresent | bool isPersonPresent(float distance); | Determines if a person is present based on distance threshold. |
| isRoomDark | bool isRoomDark(int ldrValue); | Determines if the room is dark by comparing LDR value with a threshold. |
| isHighTemperature | bool isHighTemperature(float temp); | Checks whether the current temperature is above the defined threshold. |
| controlFan | void controlFan(bool personPresent, bool highTemp); | Controls the fan relay based on person presence and temperature. |
| controlLight | void controlLight(bool personPresent, bool isDark); | Controls the light relay based on presence and ambient light. |
| printSensorData | void printSensorData(float temp, int ldr, float dist); | Prints temperature, light, and distance data to the Serial Monitor. |
| initializePins | void initializePins(); | Sets pin modes and initializes all sensors and relays at system startup. |

### 5.1 Pseudocode for Each Function:

This section presents the pseudocode for all key functions in the project. Pseudocode provides a simplified, language-neutral representation of the program's logic, making it easier to understand and explain.

1.initializePins():

Function initializePins():

Set TRIG_PIN as OUTPUT

Set ECHO_PIN as INPUT

Set FAN_RELAY as OUTPUT

Set LIGHT_RELAY as OUTPUT

Set both relays to LOW (OFF)

Start Serial Monitor

Begin DHT sensor

2. readTemperature():

Function readTemperature():

Return temperature value from DHT11 sensor in Celsius

3. readLDR():

Function readLDR():

Read analog value from LDR sensor pin

Return the light intensity value

4. getDistance():

Function getDistance():

Set TRIG_PIN to LOW

Wait 2 microseconds

Set TRIG_PIN to HIGH

Wait 10 microseconds

Set TRIG_PIN to LOW

duration ← Measure pulse time on ECHO_PIN

distance ← (duration × 0.034) / 2

Return distance in centimeters

5. isPersonPresent(distance):

Function isPersonPresent(distance):

If distance < DIST_THRESHOLD:

Return TRUE

Else:

Return FALSE

6. isRoomDark(ldrValue):

Function isRoomDark(ldrValue):

If ldrValue < LDR_THRESHOLD:

Return TRUE

Else:

Return FALSE

7. isHighTemperature(temp):

Function isHighTemperature(temp):

If temp > TEMP_THRESHOLD:

Return TRUE

Else:

Return FALSE

8. controlFan(personPresent, highTemp):

Function controlFan(personPresent, highTemp):

If personPresent AND highTemp:

Turn FAN_RELAY ON

Print "Fan: ON"

Else:

Turn FAN_RELAY OFF

Print "Fan: OFF"

9. controlLight(personPresent, isDark):

Function controlLight(personPresent, isDark):

If personPresent AND isDark:

Turn LIGHT_RELAY ON

Print "Light: ON"

Else:

Turn LIGHT_RELAY OFF

Print "Light: OFF"

10. printSensorData(temp, ldrValue, distance):

Function printSensorData(temp, ldrValue, distance):

Print "Temperature: temp °C"

Print "LDR: ldrValue"

Print "Distance: distance cm"

11. main loop():

Function loop():

temp ← readTemperature()

ldrValue ← readLDR()

distance ← getDistance()

personPresent ← isPersonPresent(distance)

isDark ← isRoomDark(ldrValue)

highTemp ← isHighTemperature(temp)

printSensorData(temp, ldrValue, distance)

controlFan(personPresent, highTemp)

controlLight(personPresent, isDark)

Wait for 2 seconds

## 5.2 Ensuring Requirements Are Met:

To ensure the system performs as expected, both functional and non-functional requirements were considered during the design and implementation phases. The following points explain how the system architecture, code structure, and hardware selection were aligned with the project requirements:

- Real-Time Responsiveness:
The system reads all sensor values (temperature, light, and distance) and updates the outputs (fan and light) every 2 seconds. This ensures near real-time responsiveness to environmental changes.

- Logical Accuracy:
The fan control logic activates the fan only when a person is present and the temperature exceeds 25°C. Similarly, the light turns ON only when a person is present and the room is dark. This dual-condition control avoids unnecessary power usage and enhances logic correctness.

- Code Modularity and Clarity:
The program is broken into modular functions such as readTemperature(), getDistance(), and controlFan(), which allows easy readability, debugging, and future modifications.

- Energy Efficiency:
Relays are activated only when necessary, ensuring that the fan and light do not operate when the room is empty or conditions do not warrant their use. This contributes to power saving and sustainability.

- User Feedback and Debugging:
Sensor values and system decisions (e.g., “Fan: ON”, “Light: OFF”) are printed to the Serial Monitor. This helps in real-time debugging, performance validation, and ensures that the system is behaving as expected.

- Hardware Safety and Isolation:
The system uses a relay module to control AC appliances, ensuring safe isolation between the microcontroller (ESP32) and high-voltage loads. The relay module is compatible with ESP32’s 3.3V logic.

- Ease of Maintenance and Tuning:
Threshold values such as temperature, distance, and LDR level are defined using #define statements. These can be easily adjusted without affecting the main program structure.

- Scalability and Future Expansion:
The ESP32 microcontroller was selected for its built-in Wi-Fi and multiple GPIO pins, allowing future expansion (e.g., adding IoT features, extra sensors, or OLED displays) without needing to change the hardware.

By addressing these points, the system not only satisfies the basic operational requirements but also ensures reliability, safety, and scalability, making it suitable for real-world deployment in smart homes, classrooms, or office spaces.

# 6. Code

// Include DHT sensor library

#include <DHT.h>

// Define DHT sensor settings

#define DHTPIN 15         // DHT11 data pin connected to GPIO15

#define DHTTYPE DHT11     // Sensor type: DHT11

DHT dht(DHTPIN, DHTTYPE); // Create DHT object

// Define pins for ultrasonic sensor

#define TRIG_PIN 5        // Trigger pin of ultrasonic sensor

#define ECHO_PIN 18       // Echo pin of ultrasonic sensor

// Define LDR sensor analog pin

#define LDR_PIN 36        // GPIO36 = ADC1_CH0 for analog read

// Define relay output pins

#define FAN_RELAY 2       // Relay controlling fan

#define LIGHT_RELAY 4     // Relay controlling light

// Define thresholds

#define TEMP_THRESHOLD 25.0    // Fan turns ON if temperature > 25°C

#define DIST_THRESHOLD 100     // Person detected if distance < 100 cm

#define LDR_THRESHOLD 1000     // Lower value means darker environment

void setup() {

Serial.begin(115200);  // Initialize serial monitor

// Set pin modes

pinMode(TRIG_PIN, OUTPUT);

pinMode(ECHO_PIN, INPUT);

pinMode(FAN_RELAY, OUTPUT);

pinMode(LIGHT_RELAY, OUTPUT);

// Turn off fan and light initially

digitalWrite(FAN_RELAY, LOW);

digitalWrite(LIGHT_RELAY, LOW);

dht.begin(); // Initialize the DHT11 sensor

}

void loop() {

// Read temperature from DHT11 sensor

float temperature = dht.readTemperature();

// Read light intensity from LDR sensor

int ldrValue = analogRead(LDR_PIN);

// Measure distance using ultrasonic sensor

float distance = getDistance();

// Print sensor readings to serial monitor for debugging

Serial.print("Temperature: ");

Serial.print(temperature);

Serial.print(" °C, LDR: ");

Serial.print(ldrValue);

Serial.print(", Distance: ");

Serial.print(distance);

Serial.println(" cm");

// Check conditions

bool personPresent = distance < DIST_THRESHOLD;     // Is a person present?

bool isDark = ldrValue < LDR_THRESHOLD;             // Is the room dark?

bool highTemp = temperature > TEMP_THRESHOLD;       // Is it hot?

// Control FAN: turn ON only if person present AND temp > 25°C

if (personPresent && highTemp) {

digitalWrite(FAN_RELAY, HIGH); // Turn fan ON

Serial.println("Fan: ON");

} else {

digitalWrite(FAN_RELAY, LOW);  // Turn fan OFF

Serial.println("Fan: OFF");

}

// Control LIGHT: turn ON only if person present AND it's dark

if (personPresent && isDark) {

digitalWrite(LIGHT_RELAY, HIGH); // Turn light ON

Serial.println("Light: ON");

} else {

digitalWrite(LIGHT_RELAY, LOW);  // Turn light OFF

Serial.println("Light: OFF");

}

delay(2000); // Wait for 2 seconds before repeating

}

// Function to calculate distance using ultrasonic sensor

float getDistance() {

digitalWrite(TRIG_PIN, LOW);      // Ensure a clean LOW pulse

delayMicroseconds(2);

digitalWrite(TRIG_PIN, HIGH);     // Send 10 µs HIGH pulse

delayMicroseconds(10);

digitalWrite(TRIG_PIN, LOW);      // End pulse

long duration = pulseIn(ECHO_PIN, HIGH); // Measure time of echo

float distance = duration * 0.034 / 2;   // Convert duration to distance in cm

return distance;

}

# 7.Testing and Results

Thorough testing was conducted to verify the functional correctness, response time, and reliability of the smart fan and light control system. Each component and logic block was individually validated and then tested in an integrated manner.

## 7.1 Testing Methodology

The testing approach included:

- Unit Testing: Individual sensor readings (temperature, distance, and light) were validated through Serial Monitor output.

- Integration Testing: Combined logic for controlling the fan and light based on sensor inputs was tested under different environmental conditions.

- Threshold Tuning: Temperature, distance, and LDR thresholds were adjusted and tested to ensure optimal switching points.

- Stress Testing: The system was run continuously for multiple hours to ensure stability and performance over time.

| Test Case | Input Conditions | Expected Output | Observed Output | Status |
| --- | --- | --- | --- | --- |
| Person not present, normal light, normal temp | Distance > 100 cm, LDR > 1000, Temp < 25°C | Fan: OFF, Light: OFF | Fan: OFF, Light: OFF | ✅ Passed |
| Person present, bright room, high temp | Distance < 100 cm, LDR > 1000, Temp > 25°C | Fan: ON, Light: OFF | Fan: ON, Light: OFF | ✅ Passed |
| Person present, dark room, normal temp | Distance < 100 cm, LDR < 1000, Temp < 25°C | Fan: OFF, Light: ON | Fan: OFF, Light: ON | ✅ Passed |
| Person present, dark room, high temp | Distance < 100 cm, LDR < 1000, Temp > 25°C | Fan: ON, Light: ON | Fan: ON, Light: ON | ✅ Passed |
| Person not present, dark room, high temp | Distance > 100 cm, LDR < 1000, Temp > 25°C | Fan: OFF, Light: OFF | Fan: OFF, Light: OFF | ✅ Passed |
| Continuous operation test (6 minutes runtime) | Varying inputs every 30 seconds | Stable and timely switching | No crash or delay | ✅ Passed |

## 7.2 Key Results:

## The system was able to accurately detect environmental conditions and control the appliances as expected.

## Response time from sensor reading to relay activation was under 2 seconds consistently.

## The relay outputs remained stable with no unintended switching during prolonged operation.

## Sensor readings remained accurate and reliable, even after extended usage.

## The system maintained logical correctness across all possible input combinations.

## 7.3 Conclusion from Testing:

The system met all functional requirements and performed reliably in real-world conditions. The smart logic used to control the fan and light ensured energy-efficient operation based on actual room conditions. All results validate the system’s readiness for deployment in small-scale environments such as homes, offices, and classrooms.

# 8. Future Work:

While the current system successfully achieves smart and automated control of fan and light based on real-time environmental conditions, there is significant potential to expand and enhance the project further. The following are some proposed areas for future improvement:

1. IoT Integration

- Enhancement: Add Wi-Fi-based monitoring and control using platforms like Blynk, Firebase, or MQTT.

- Benefit: Users can remotely monitor room conditions and control appliances using a mobile app or web dashboard.

2. Data Logging and Analytics

- Enhancement: Log temperature, light, and presence data over time using an SD card or cloud database.

- Benefit: Enables energy usage analysis, predictive control, and environment monitoring trends.

3. Machine Learning-Based Control

- Enhancement: Implement AI or ML algorithms to predict usage patterns and automate switching more intelligently.

- Benefit: Personalized automation based on user behavior and environmental history.

4. Solar Power and Energy Optimization

- Enhancement: Integrate solar panels with battery storage for powering the system and connected appliances.

- Benefit: Promotes sustainability and enables the system to run in off-grid conditions.

5. Modular Expansion with More Sensors

- Enhancement: Add more environmental sensors like:

  - PIR sensor for motion detection

  - MQ gas sensor for air quality

  - Sound sensor for noise level

- Benefit: Enables multi-parameter control and smarter automation.

6. OLED or LCD Display Integration

- Enhancement: Add a small screen to display live temperature, light level, and status of fan/light.

- Benefit: Makes the system user-friendly and informative without needing external devices.

7. Voice Assistant and Smart Home Integration

- Enhancement: Connect the system to Google Assistant or Alexa via NodeMCU/ESP32 and IFTTT.

- Benefit: Hands-free control using voice commands enhances accessibility and convenience.

8. Over-The-Air (OTA) Updates

- Enhancement: Add support for OTA firmware updates using ESP32's Wi-Fi capabilities.

- Benefit: Enables remote code updates without physical access to the device.

9. Multi-Room or Multi-Zone Control

- Enhancement: Expand the system to control multiple fans/lights in different rooms using a central controller.

- Benefit: Makes the system scalable and suitable for larger homes or buildings.

10. Safety and Emergency Features

- Enhancement: Add smoke detection or emergency shutdown in case of abnormal temperature spikes or sensor failures.

- Benefit: Increases reliability and ensures safe operation in all conditions.

By implementing these future enhancements, the project can evolve into a fully automated, intelligent, and remotely accessible smart home system, offering better energy management, user control, and safety.